VIETNAM-KOREA UNIVERSITY OF INFORMATION AND COMMUNICATIONS TECHNOLOGY

FACULTY OF COMPUTER ENGINEERING AND ELECTRONICS

SUMMARY OF GRADUATION THESIS

AIoT-Based Weather Forecasting System Using Machine Learning

Student's Full Name	: Thai Van Hoa

Class	: 21IR

Major	: Information Technology

Specialization	: IoT & Robotics

Instructor	: Ph.D. Vuong Cong Dat

Da Nang, December 2025

VIETNAM-KOREA UNIVERSITY OF INFORMATION AND COMMUNICATIONS TECHNOLOGY

FACULTY OF COMPUTER ENGINEERING AND ELECTRONICS

SUMMARY OF GRADUATION THESIS

AIoT-Based Weather Forecasting System Using Machine Learning

Student's Full Name	: Thai Van Hoa

Class	: 21IR

Major	: Information Technology

Specialization	: IoT & Robotics

Instructor	: Ph.D. Vuong Cong Dat

Da Nang, December 2025

# PREAMBLE

## 1. Reason for Choosing the Topic

In recent years, weather and environmental conditions have become increasingly unpredictable due to climate change and rapid urbanization. Extreme heat, sudden rainfall, large humidity fluctuations, and deteriorating air quality have directly affected public health, transportation safety, and daily activities. For areas such as schools, public parks, and densely populated neighborhoods, continuous monitoring of environmental parameters and early weather forecasting is essential to reduce risks and support proactive decision-making.

However, in many locations, people often rely only on generalized online weather information, which may not accurately represent the microclimate at a specific site (e.g., inside a school campus or a public park). In addition, traditional monitoring methods if available are often manual, discontinuous, and lack long-term data storage, making it difficult to analyze trends, detect anomalies early, or provide timely warnings.

With the advancement of the Internet of Things (IoT), low-cost sensor nodes such as ESP32 enable continuous measurement of key parameters including temperature, humidity, pressure, CO₂, dust/PM2.5, and air-quality indicators. Nevertheless, monitoring alone is not sufficient; users increasingly need predictive capabilities to anticipate upcoming conditions (e.g., rainfall likelihood, temperature/humidity trends, or air-quality risks) for planning and prevention.

Meanwhile, Machine Learning has demonstrated strong performance in time-series forecasting by learning trends and seasonal patterns from historical data. When IoT sensor data is continuously collected, stored in a database (e.g., MySQL), and integrated with Weather API data, it becomes feasible to build a complete AIoT pipeline that supports both real-time monitoring and short-term forecasting with measurable accuracy.

For these reasons, I chose the graduation project “AIoT-Based Weather Forecasting System Using Machine Learning” to develop an integrated system for data acquisition, storage and management, model training for forecasting, and practical visualization for end users.

# CHAPTER I : OVERVIEW

## 1.1 Background and Motivation

### 1.1.1 Current context and practical needs

In recent years, climate change and rapid urbanization have made weather and environmental conditions more volatile and less predictable. Extreme heat, sudden heavy rainfall, abrupt humidity changes, and increasing fine-dust pollution negatively impact public health as well as daily activities. In sites such as schools, public parks, and densely populated areas, real-time monitoring of environmental conditions is increasingly necessary to support timely warnings and practical decisions (e.g., outdoor activity scheduling, limiting exposure to fine dust, or preparing for rain).

### 1.1.2 Motivation for combining IoT and Machine Learning

The advancement of the Internet of Things (IoT) enables the deployment of low-cost sensor nodes such as ESP32 for continuous, automated, and scalable data acquisition. Key parameters for environmental assessment and forecasting include temperature, humidity, pressure, CO₂, dust/PM2.5, and related indicators. With a gateway such as Raspberry Pi and data-flow processing via Node-RED, the collected data can be normalized and stored centrally in MySQL for long-term querying and analytics.

Nevertheless, real-time monitoring only reflects the “current state,” while practical users often require forecasting to take proactive actions: anticipating temperature/humidity changes, rainfall likelihood, or air quality trends. This is where Machine Learning becomes crucial. With sufficient historical data, ML models can learn trends, seasonal patterns, and relationships among variables to produce short-term forecasts such as 24-hour and 7-day predictions.

### 1.1.3 System and data objectives

- Build an ESP32-based sensing system to measure environmental parameters: temperature, humidity, pressure, CO₂, and dust/PM2.5.

- Collect additional Weather API data: wind speed, wind direction, rainfall, and UV index.

- Deploy Raspberry Pi + Node-RED as a gateway to process data streams and store data in MySQL, building structured historical datasets for analytics and training.

# CHAPTER II : THEORETICAL BASIS

## 2.1 Internet of Things (IoT)

### 2.1.1 Introduction to IoT

The Internet of Things (IoT) represents a paradigm shift in how physical devices interact with digital systems. IoT encompasses a network of physical objects embedded with sensors, software, and connectivity capabilities that enable them to collect, exchange, and act upon data without human intervention. According to recent estimates, the number of connected IoT devices worldwide has surpassed 15 billion and continues to grow exponentially.

Figure 2. 1 Internet of Things

### 2.1.2 IoT Architecture (4 layers)

A comprehensive IoT system is typically structured in four hierarchical layers, each serving distinct functions:

1. Perception Layer (Sensing Layer) This foundational layer consists of physical sensors and actuators that interact directly with the environment. In weather monitoring systems, this includes temperature sensors, humidity sensors, barometric pressure sensors, and other environmental monitoring devices. The perception layer is responsible for data acquisition and preliminary signal processing.

2. Network Layer (Transmission Layer) The network layer handles data transmission from sensing devices to processing systems. This layer employs various communication protocols and technologies such as LoRa, Wi-Fi, Zigbee, or cellular networks. The choice of technology depends on factors including transmission range, power consumption, data rate requirements, and deployment environment.

3. Processing Layer (Middleware Layer) This layer manages data storage, processing, and analysis. It typically includes cloud servers, databases, and computational resources that handle large volumes of sensor data. The processing layer performs functions such as data aggregation, filtering, analytics, and machine learning model execution.

4. Application Layer The topmost layer provides user interfaces and application-specific services. This includes web dashboards, mobile applications, alert systems, and APIs that enable end-users to access processed information and insights derived from sensor data.

Figure 2. 2 4 layers of IOT

## 2.2 Communication Protocols

### 2.2.1 MQTT Protocol

MQTT (Message Queuing Telemetry Transport) is a message transmission protocol based on the Publish/Subscribe communication model, suitable for remote data transport.

Figure 2. 3 MQTT Protocol

### 2.2.2 TCP/IP Protocol

Transmission Control Protocol/Internet Protocol (TCP/IP) forms the foundation of Internet communication and provides the underlying transport mechanism for MQTT and other application-layer protocols.

TCP/IP Protocol Suite Layers:

- Application Layer: Hosts protocols like HTTP, MQTT, FTP that applications use directly

- Transport Layer: TCP provides reliable, connection-oriented data delivery with error checking and flow control

- Network Layer: IP handles routing and addressing, enabling data packets to reach their destination across networks

- Link Layer: Manages physical network connections and hardware addressing

MQTT over TCP/IP

Figure 2. 4 : TCP/IP

MQTT operates at the application layer and relies on TCP for reliable data transmission. The typical communication flow involves:

TCP's reliability mechanisms (acknowledgments, retransmission, ordering) complement MQTT's QoS levels, ensuring dependable data delivery in IoT systems. For resource-constrained devices or networks with high packet loss, alternatives like MQTT-SN (over UDP) may be considered.

## 2.3 LoRa Technology

### 2.3.1 LoRa Introduction

Long Range (LoRa) is a wireless modulation technique designed for long-distance, low-power communication in IoT applications. Developed by Semtech Corporation, LoRa uses Chirp Spread Spectrum (CSS) modulation to achieve exceptional range and penetration characteristics while maintaining minimal power consumption.

LoRa operates in unlicensed ISM (Industrial, Scientific, and Medical) frequency bands, which vary by region: 868 MHz in Europe, 915 MHz in North America, and 433 MHz in Asia. This project utilizes the 433 MHz band, which offers excellent range characteristics and building penetration in the Vietnamese deployment environment.

LoRaWAN Quick Overview

Figure 2. 5 : LoRa & LoRaWAN

LoRaWAN (Long Range Wide Area Network) is a network protocol built on top of LoRa physical layer technology. While this project uses point-to-point LoRa communication, LoRaWAN provides additional features for large-scale deployments:

- Network architecture with end-devices, gateways, network servers, and application servers

- Device classes (A, B, C) for different power/latency trade-offs

- Built-in security with AES-128 encryption

- Adaptive data rate (ADR) for optimization

### 2.3.2 Module E32-433T20D

The E32-433T20D is a LoRa transceiver module manufactured by Ebyte, featuring the Semtech SX1278 LoRa chip. This module is particularly suitable for weather monitoring applications due to its robust specifications and low power consumption.

Figure 2. 6 : E32-433T20D

Key Specifications:

Operating Modes: The module supports multiple operating modes to optimize power consumption:

- Normal Mode: Full functionality for transmitting and receiving

- Wake-Up Mode: Periodically checks for incoming messages while conserving power

- Power-Saving Mode: Ultra-low power consumption with configurable wake-up periods

- Sleep Mode: Minimal power consumption with wake-up on external trigger

Battery Operation Strategy For battery-powered sensor nodes, the module operates primarily in power-saving mode, waking periodically to collect sensor data and transmit to the gateway. With optimized duty cycling (e.g., 1-minute transmission intervals with sleep between), a node can operate for 1-2 years on battery power.

### 2.3.4 Comparison of LoRa with Wi-Fi/4G

Different wireless technologies suit different IoT application requirements. Here's a comparative analysis:

| Feature | LoRa | Wi-Fi | 4G/LTE |
| --- | --- | --- | --- |
| Range | 2-15 km | 50-100 m | 2-10 km |
| Data Rate | 0.3-50 kbps | 150+ Mbps | 1-100+ Mbps |
| Power Consumption | Very Low | Medium-High | High |
| Battery Life | Years | Days-Weeks | Days |
| Infrastructure Cost | Low | Medium | High (subscription) |
| Deployment | Private network | Requires router | Network coverage |
| License | Unlicensed ISM | Unlicensed | Licensed spectrum |
| Best For | Long-range sensors | High-bandwidth local | Mobile, high-speed |

Table 2. 1: LoRa Comparison Table

Why LoRa for Weather Monitoring:

- Extended range enables sparse gateway deployment covering large areas

- Ultra-low power consumption allows long-term battery operation in remote locations

- Low infrastructure cost for private network deployment

- Sufficient data rate for periodic transmission of small sensor data packets

## 2.4 Machine Learning for Weather Forecasting

### 2.4.1 ML Overview for Forecasting

Machine Learning (ML) has emerged as a powerful approach for weather forecasting, complementing traditional numerical weather prediction models. ML algorithms excel at identifying complex patterns in historical data and generating predictions based on learned relationships.

ML Workflow for Weather Prediction:

- Data Collection: Gather historical weather observations

- Data Preprocessing: Clean, normalize, and engineer features

- Model Training: Train algorithms on historical patterns

- Validation: Evaluate model performance on test data

- Deployment: Generate real-time forecasts from current conditions

- Monitoring: Continuously assess and improve model accuracy

### 2.4.2 Time Series Forecasting

Time series forecasting involves predicting future values based on previously observed data points ordered chronologically. Weather data is inherently a time series with distinct characteristics that influence modeling approaches.

Time Series Characteristics:

1. Trend Long-term increase or decrease in the data. For example, gradual temperature changes across seasons or climate trends over years.

2. Seasonality Regular patterns that repeat at fixed intervals. Daily temperature cycles (warm days, cool nights) and seasonal variations (summer heat, winter cold) are classic examples in weather data.

3. Cyclicity Patterns that repeat at irregular intervals, such as multi-year climate oscillations (El Niño/La Niña effects).

### 2.4.3 Prophet Model

Prophet is an open-source forecasting library developed by Facebook's Core Data Science team, designed for forecasting time series data with strong seasonal patterns and multiple seasons of historical data.

Figure 2. 7 : Prophet model

Principle of Operation

Prophet decomposes time series into three main components using an additive model:

y(t) = g(t) + s(t) + h(t) + εₜ

Where:

- g(t): Trend function modeling non-periodic changes (piecewise linear or logistic growth)

- s(t): Seasonal component representing periodic changes (Fourier series)

- h(t): Holiday effects and irregular events

- εₜ: Error term representing idiosyncratic changes

Key Features:

Automatic Seasonality Detection Prophet automatically detects daily, weekly, and yearly seasonal patterns using Fourier series. For weather data, this captures diurnal temperature cycles and annual seasonal variations.

Trend Changepoints The model automatically identifies points where the time series trend changes, allowing for flexible trend modeling without manual intervention.

Robust to Missing Data Prophet handles missing values and outliers gracefully, making it suitable for real-world sensor data that may have gaps.

Interpretable Parameters Model components (trend, seasonality, holidays) are interpretable, enabling understanding of what drives predictions.

### 2.4.4: LightGBM Model

LightGBM (Light Gradient Boosting Machine) is a gradient boosting framework developed by Microsoft that uses tree-based learning algorithms. It is highly efficient and has become popular for both classification and regression tasks, including time series forecasting.

Gradient Boosting Principle

Gradient Boosting builds an ensemble of decision trees sequentially, where each new tree corrects errors made by the existing ensemble.

Mathematical Formulation:

F(x) = F₀(x) + Σ γₘ hₘ(x)

Where:

- F(x) is the final prediction

- F₀(x) is the initial prediction (baseline)

- hₘ(x) is the m-th decision tree

- γₘ is the learning rate (shrinkage parameter)

## 2.5 Web Technologies and Data Processing

### 2.5.1 Node-RED

Node-RED is a flow-based development tool originally developed by IBM for wiring together hardware devices, APIs, and online services. It provides a browser-based visual editor that makes it easy to create data processing workflows using a wide range of pre-built nodes.

Figure 2. 8 :Node RED

Advantages:

- Visual programming reduces development time

- Extensive library of pre-built nodes for common tasks

- Real-time data processing with low latency

### 2.5.2 MySQL Database

MariaDB and MySQL are two popular open-source Relational Database Management Systems (RDBMS) that are based on the Structured Query Language (SQL) for managing tabular data. MariaDB is a fork from MySQL (due to ownership concerns after Oracle bought MySQL), so they are highly compatible and easy to migrate between the two parties. By 2025, MariaDB is generally faster at some tasks (bulk inserts, replication, analytic queries), supports more storage engines and powerful community features, while MySQL has better native JSON support and enterprise from Oracle.

Figure 2. 9 : MySQL Database

### 2.5.3 FastAPI Framework

FastAPI is a modern, fast (high-performance) web framework for building APIs with Python 3.7+ based on standard Python type hints. It serves as the backend API layer connecting the database to web/mobile clients and integrating machine learning models.

Figure 2. 10 : FastAPI

Key Features:

1. High Performance FastAPI is one of the fastest Python frameworks, comparable to NodeJS and Go, built on Starlette for web parts and Pydantic for data validation.

2. Automatic API Documentation FastAPI automatically generates interactive API documentation (Swagger UI and ReDoc) based on OpenAPI standards, making API testing and integration straightforward.

## 2.6 The hardware

### 2.6.1 Raspberry Pi 4B

The Raspberry Pi 4B is a small-sized computer that runs on the Linux operating system. This compact computer is primarily used to execute large programs to achieve fast output signals.

Figure 2. 11: Raspberry Pi 4B

### 2.6.2 Esp32

The ESP32 is a powerful, versatile module that supports Wi-Fi, Bluetooth, and BLE, making it ideal for a wide range of projects. It is well-suited for applications requiring networking, low power consumption, or demanding tasks such as voice encoding, music playback, and MP3 decoding.

ESP32 has a dual-core CPU, with each core being independently controllable, and the CPU clock frequency is adjustable from 80 MHz to 240 MHz.

Figure 2. 12: Esp32 Wroom Pin Diagram

### 2.6.3 MQ-135

MQ-135 is a gas sensor from the MQ series, based on a SnO₂ sensing element whose conductivity varies with gas concentration.

Figure 2. 13 :MQ-135

### 2.6.4 DHT11 Temperature and Humidity Sensor

The DHT11 temperature and humidity sensor is widely used due to its low cost and ease of data retrieval via one-wire communication.

Figure 2. 14 DHT11 Sensor

### 2.6.5 BMP280 + ATH20

BMP280 is a barometric pressure sensor with an integrated temperature measurement. AHT20 is a digital temperature and humidity sensor designed for stable real-world measurements. It communicates via I²C

Figure 2. 15 :BMP280+ATH20

### 2.6.6 OLED 0.96

The 0.96-inch OLED (commonly based on the SSD1306 controller) is a monochrome display (often white/blue) that provides high contrast, good readability,

Figure 2. 16 : 0.96-inch OLED

### 2.6.7 GP2Y1010AU0F

GP2Y1010AU0F is an optical dust sensor based on light scattering. It integrates an IR LED and a photodiode

Figure 2. 17 : GP2Y1010AU0F

### 2.7.8  Lora E32 433T20D

LoRa E32-433T20D (Ebyte) is a long-range wireless module based on LoRa technology (powered by the Semtech SX1278) with a TTL-level UART interface operating in transparent serial transmission mode

Figure 2. 18 : LoRa E32-433T20D

# CHAPTER III : SYSTEM ANALYSIS AND DESIGN

## 3.1 System Overview

The Raspberry Pi 4B and ESP32 sensor nodes (one or multiple nodes) operate within the same networking infrastructure (Wi-Fi; or Wi-Fi for the gateway and LoRa between nodes and the gateway depending on deployment).

Each ESP32 Sensor Node interfaces with environmental sensors (e.g., temperature, humidity, pressure, CO₂, particulate/dust) and samples data periodically.

Figure 3. 1 : System Overview

The ESP32 devices function as MQTT clients:

- They publish sensor measurements to structured topics (by node and by sensor type).

- They subscribe to control topics (when actuators such as relays/fans are included) and execute received commands.

On the Raspberry Pi, the system deploys a backend service (e.g., FastAPI) and a machine learning module to:

- Extract historical time-series data from the database.

- Train/update forecasting models (e.g., 24-hour and 7-day predictions for key environmental metrics).

## 3.2 Overall System Architecture

### 3.2.1 System Block Diagram

Figure 3. 2 : System Block Diagram

### 3.2.2 Algorithm Flowcharts

Figure 3. 3 : Algorithm Flowcharts

## 3.3 Hardware Design

### 3.3.1 Schematic Design

Figure 3. 4 :Schematic

### 3.3.2 PCB Design

PCB design focuses on analog measurement stability

Figure 3. 5 :3D PCB

### 3.3.3 Finished Products

The device is an enclosure-based environmental monitoring module designed to collect and display real-time air parameters. On the front panel, the main components include a 0.96" OLED display for data visualization, a push button for switching modes/viewing different parameter pages, a status LED indicating power/operation, and integrated environmental sensors.

An antenna is mounted at the top, making the device suitable for wireless data transmission scenarios (e.g., Wi-Fi/LoRa, depending on the design). This layout improves visibility, ease of operation, and maintenance, and it meets the requirements for environmental monitoring in classrooms, corridors, school campuses, or other public areas

Figure 3. 6 : Finished Products

# CHAPTER IV : RESULTS AND EVALUATION

## 4.1 System implementation results

After the design and implementation phases, the AIoT system was deployed following a multi-layer architecture including IoT sensing nodes, LoRa communication, an edge/gateway layer on Raspberry Pi (MQTT Broker + Node-RED + MySQL), and the application layer (FastAPI + Web Dashboard). During operation, environmental measurements (temperature, humidity, pressure, CO₂, dust/PM2.5, AQI) are periodically collected, packaged into timestamped records, and persistently stored in MySQL. In addition, the system integrates supplementary weather data from a Weather API (wind speed/direction, rainfall, UV index) to enrich the meteorological context and potentially improve forecasting performance.

Figure 4. 1 : Interface Forecasting

On the dashboard, users can monitor real-time readings, inspect time-series trend charts, and access data management functions (time-range filtering, quick queries, exporting, and conditional deletion). With centralized and consistent storage, the dataset for model training preserves time-series continuity, minimizes manual processing, and facilitates systematic evaluation and comparison of forecasting models.

## 4.2 Forecasting evaluation

### 4.2.1 Evaluation methodology and metrics

Figure 4. 2 : Training model interface

testing subsets in chronological order to avoid data leakage. Predictions are then compared to the ground-truth measurements over the same test horizon. The main metrics include MAE (Mean Absolute Error), RMSE (Root Mean Squared Error), and R² (coefficient of determination).

## 4.3 Compare models

Figure 4. 3 : Compare Models

Prophet achieves an average accuracy of 39.07%, while LightGBM reaches 71.10%, indicating that LightGBM delivers substantially better overall forecasting performance in this evaluation.

# CONCLUSION AND DEVELOPMENT DIRECTION

## 1. Conclusion

The project “AIoT-Based Weather Forecasting System Using Machine Learning” successfully designed and deployed an end-to-end AIoT system for environmental monitoring and forecasting using a multi-layer architecture. The system achieves the primary objectives: (i) collecting environmental data from IoT sensing nodes; (ii) enabling long-range uplink via LoRa to a gateway; (iii) performing edge-layer integration on Raspberry Pi using MQTT, Node-RED, and centralized persistence in MySQL; (iv) providing a web dashboard for monitoring, visualization, and data administration; and (v) integrating training/forecasting modules with Prophet and LightGBM for 24-hour and 7-day horizons. The practical deployment demonstrates feasibility for school/park scenarios that require stable long-range communication and reduced dependence on cloud infrastructure.

In terms of contributions, the project delivers a closed-loop pipeline from data acquisition → storage → training → forecasting → visualization, with modular design and scalability. Moreover, the comparative analysis between Prophet (time-series modeling) and LightGBM (machine learning boosting) clarifies strengths and weaknesses of each approach under environmental sensor data conditions, supporting model selection for real-world requirements..

## 2. Limitations of the topic

Despite meeting the stated objectives, several limitations remain. First, forecasting quality strongly depends on the length and continuity of historical time-series data; when the dataset is short, missing samples, or noisy due to environmental/device factors, prediction errors can increase substantially. Second, sensor-related issues (drift over time, installation placement, shielding conditions, power stability) may reduce input reliability and cause the model to learn from a distorted distribution. Third, the current scope focuses mainly on monitoring and forecasting basic environmental variables; advanced functions such as anomaly detection, context-aware early warnings, or automated model tuning have not been fully implemented within the project scope.